Diagnostic Chart for Air Conditioning System

The diagnostic chart can help to understand how different conditions can affect the entire system. The chart gives symptoms that normally are associated with the conditions that are listed.

Use the Troubleshooting Charts and the Flow Charts for Diagnosing the System to locate the source of a problem and systematically examine and repair the problem.

| Condition | Low Side Pressure | High Side Pressure | Sight Glass | Suction Line | ReceiverDrier | Liquid Line | Discharge Line | Discharge Air |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LACK OF REFRIGERANT | Very low | Very low | Clear | Slightly | Slightly | Slightly | Slightly | Warm |
|  |  |  |  | cool | warm | warm | warm |  |
| LOSS OF | Low | Low | Bubbles | Cool | Warm to | Warm | Warm to | Slightly cool |
| REFRIGERANT |  |  |  |  | hot |  | hot |  |
| COMPRESSOR |  |  |  |  |  |  |  |  |
| FAILURE | High | Low | Clear | Cool | Warm | Warm | Warm | Slightly cool |
|  |  |  | Clear to | Slightly |  |  |  |  |
| CONDENSER | High | High | occasional |  | Hot | Hot | Hot | Warm |
|  |  |  |  | cool to |  |  |  |  |
| MALFUNCTION |  |  | bubbles | warm |  |  |  |  |
|  |  |  |  | Cold — |  |  |  |  |
| EXPANSION |  | High or |  | sweating |  |  |  |  |
| VALVE STUCK | High | Normal | Clear | or | Warm | Warm | Hot | Slightly cool |
| OPEN |  |  |  | frosting heavily |  |  |  |  |

Cold — sweating

EXPANSION or

VALVE STUCK Low Low Clear frosting Warm Warm Hot Slightly cool

CLOSED heavily

at valve

inlet

| RESTRICTION BETWEEN CONDENSER AND EXPANSION VALVE | Low | Low | Clear | Cool | Cool or sweating or frosting | Cool or sweating or frosting | Hot to point of restriction | Slightly cool |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RESTRICTION BETWEEN COMPRESSOR AND CONDENSER | High | High, normal, or low | Clear | Slightly cool to warm | Warm or hot | Warm or hot | Hot | Warm |

|  | Cool, Cool (see |
| --- | --- |
| NORMAL | Normal Normal Clear possible Warm Warm Hot specification light on page 63) |

sweat