HVAC/R TRAINING — TWO-DAY CLASS

Bobcat A/C System: Diagnosis, Recovery & Recharge

8:00 AM – 4:00 PM  |  R-134a Systems  |  5 Technicians  |  5 Stations  |  1 Recovery Machine per Station

| 5 Technicians | 5 Stations | 120 Min per Station | 3 Types Faults Planted | 2 Class Days |
| --- | --- | --- | --- | --- |

| DAY 1 Stations 1 & 2 Opening Session + 2 Full Rotations | DAY 2 Stations 3, 4 & 5 3 Full Rotations + Closing Debrief |
| --- | --- |

Attendance flexibility: If fewer than 5 techs attend on either day, pair techs on machines. Teams of two take turns performing each step — one acts, one observes — then swap roles at the next station. The schedule and rotation matrix still apply.

Step-by-Step Station Breakdown (120 min per station)

Every technician performs all 7 steps at every station in sequence. Steps are identical across all 5 machines — the tech must determine the machine's condition through their own diagnosis. The reset step (Step 7) re-introduces the original fault so the next team encounters the same scenario.

| # | Step | Time | What to Do & Document |
| --- | --- | --- | --- |
| 1 | Test Operate | 15 min | Start machine and run A/C. Observe and record cab cooling, airflow, compressor cycling, and any abnormal sounds. Document initial impressions before connecting gauges. |
| 2 | Diagnose | 20 min | Connect manifold gauge set to high and low service ports. Record static and running pressures. Compare to R-134a normal operating range. Identify fault type: undercharge, overcharge, or restriction. Record diagnosis on worksheet before proceeding. |
| 3 | Recover Refrigerant | 28 min | Connect recovery machine (dedicated unit at this station). Open manifold valves and begin recovery. Monitor recovery cylinder weight — do not exceed 80% capacity. Confirm system reaches vacuum on gauges before disconnecting. Log refrigerant quantity recovered. |
| 4 | Dehydrate (Evacuation) | 15 min | Connect micron gauge to system. Start vacuum pump and observe pull-down on the micron gauge display. Watch the curve drop toward target depth. Instructor will show pre-evacuated reference reading for comparison. Document target micron level on worksheet. (Full field evacuation: 30–45 min — this step is abbreviated for training.) |
| 5 | Re-fill / Recharge | 15 min | Using refrigerant scale, weigh in R-134a to OEM specification. Charge liquid or vapor side per system requirements. Monitor gauge pressures as charge builds. Do not exceed spec. Document total charge weight on worksheet. |
| 6 | Verify Operation | 12 min | Restart machine and run A/C system. Recheck high and low side pressures. Confirm cab cooling restored. Calculate superheat and subcooling and verify within acceptable range. Sign off on repair worksheet. |
| 7 | Reset / Re-introduce Fault | 15 min | Following the station's fault card, re-introduce the original fault condition (e.g., remove refrigerant to undercharge, re-restrict filter-drier, add excess charge). Confirm fault is active. Notify instructor. Prepare station for next team. |

★  Dehydration note: The 15-minute Step 4 is an abbreviated learning exercise. Techs observe the vacuum pull-down curve and document the target depth. Full field-standard evacuation (30–45 min to target micron level with decay test) is required before any machine returns to field service.

★  Reset note: Step 7 is a critical skill. Techs must consult their station fault card and precisely re-introduce the fault — not just approximately. Instructor should verify the fault is correctly set before calling rotation.

DAY 1 SCHEDULE

Stations 1 & 2  |  Opening Session + 2 Full Rotations

| Time | Activity | Notes / Detail |
| --- | --- | --- |
| 8:00 – 9:00 AM | Opening Session | Company mission & values ¦ Tech introductions ¦ Safety briefing: R-134a handling, EPA 608 compliance, PPE requirements, recovery machine operation |
| 9:00 – 11:00 AM | Station 1 — All Steps | Test Operate → Diagnose → Recover → Dehydrate → Re-fill → Verify → Reset (120 min) |
| 11:00 – 11:05 AM | ⟳ Rotate to Station 2 | Instructor calls time. Techs confirm reset is complete, then move to next machine. |
| 11:05 – 11:20 AM | Morning Break | 15 minutes |
| 11:20 AM – 1:20 PM | Station 2 — All Steps | Test Operate → Diagnose → Recover → Dehydrate → Re-fill → Verify → Reset (120 min) |
| 1:20 – 1:50 PM | Lunch Break | 30 minutes |
| 1:50 – 2:05 PM | Afternoon Break / Buffer | 15 minutes — use as buffer if Station 2 runs long, or as additional rest |
| 2:05 – 4:00 PM | Day 1 Wrap & Preview | Group check-in: What did you find at each machine? Common questions. Preview Day 2 stations. Equipment check and securing. |

Day 1 buffer: The afternoon break (1:50–2:05 PM) doubles as a schedule buffer. If Station 2 runs long due to new techs needing extra time, the break absorbs the overage. The Day 1 wrap session can be shortened accordingly.

DAY 2 SCHEDULE

Stations 3, 4 & 5  |  3 Full Rotations + Closing Debrief

| Time | Activity | Notes / Detail |
| --- | --- | --- |
| 8:00 – 8:15 AM | Day 2 Opening | Recap Day 1 findings ¦ Answer overnight questions ¦ Confirm stations are reset and ready |
| 8:15 – 10:05 AM | Station 3 — All Steps | Test Operate → Diagnose → Recover → Dehydrate → Re-fill → Verify → Reset (110 min) |
| 10:05 – 10:10 AM | ⟳ Rotate to Station 4 | Instructor calls time. Techs confirm reset, then move. |
| 10:10 – 10:25 AM | Morning Break | 15 minutes |
| 10:25 AM – 12:25 PM | Station 4 — All Steps | Test Operate → Diagnose → Recover → Dehydrate → Re-fill → Verify → Reset (120 min) |
| 12:25 – 12:30 PM | ⟳ Rotate to Station 5 | Instructor calls time. Techs confirm reset, then move. |
| 12:30 – 1:00 PM | Lunch Break | 30 minutes |
| 1:00 – 3:00 PM | Station 5 — All Steps | Test Operate → Diagnose → Recover → Dehydrate → Re-fill → Verify → Reset (120 min) |
| 3:00 – 3:15 PM | Afternoon Break | 15 minutes |
| 3:15 – 4:00 PM | Closing Debrief | Group diagnostic review ¦ Common errors & lessons learned ¦ Dehydration & EPA 608 Q&A ¦ Field application discussion ¦ Paperwork & sign-offs |

Day 2 note: Station 3 is allotted 110 minutes (vs. 120) to allow the debrief to start at 3:15 PM. By Day 2, Station 3, techs will be more familiar with the process and the shorter window should not be a problem. If a team is still working at the 110-minute mark, instructor may grant a 5-minute grace period and trim the following break.

Machine Fault Reference — Instructor Only

Do not distribute to technicians before or during the class. Each station has a laminated fault card for tech reference during Step 7 reset only.

| Station | Day | Fault | Instructor Notes / Reset Method |
| --- | --- | --- | --- |
| Machine 1 | Day 1 | Undercharge / Leak | Low suction pressure, high superheat. Tech locates leak scenario. Re-introduce by bleeding refrigerant to undercharge spec. |
| Machine 2 | Day 1 | Restriction (Filter-Drier / Metering Device) | Low suction and discharge pressure, possible frosted drier or TXV. Re-introduce by re-installing restricted component. |
| Machine 3 | Day 2 | Overcharge | High head pressure, low superheat, possible slugging. Re-introduce by adding excess R-134a above OEM spec. |
| Machine 4 | Day 2 | Undercharge / Leak (repeat) | Subtle variation from Machine 1 — reinforces diagnosis confidence. Re-introduce same way. |
| Machine 5 | Day 2 | Known-Good (Baseline) | Normal pressures and operation. Tech must confidently confirm no fault present. No re-introduction needed — recharge to spec and verify. |

Tech Rotation Matrix

All 5 techs rotate simultaneously on the instructor's timer. Each tech visits every machine exactly once across the two days. Stations 1–2 are Day 1; Stations 3–5 are Day 2.

| Tech | Stn 1 Day 1 | Stn 2 Day 1 | Stn 3 Day 2 | Stn 4 Day 2 | Stn 5 Day 2 | Machines Visited |
| --- | --- | --- | --- | --- | --- | --- |
| Tech 1 | M1 | M2 | M3 | M4 | M5 | All 5 |
| Tech 2 | M2 | M3 | M4 | M5 | M1 | All 5 |
| Tech 3 | M3 | M4 | M5 | M1 | M2 | All 5 |
| Tech 4 | M4 | M5 | M1 | M2 | M3 | All 5 |
| Tech 5 | M5 | M1 | M2 | M3 | M4 | All 5 |

Blue = Day 1 station  |  Green = Day 2 station

Closing Debrief Guide — Day 2  (3:15 – 4:00 PM)

Allow techs to share their findings and reasoning before the instructor confirms answers. The goal is to surface misunderstandings and reinforce field-ready thinking.

Diagnostic Review

- Go around the room: what fault did each tech identify at each machine? Did anyone get a different reading on the same machine?

- Known-good machine (Machine 5): who suspected a fault? What pressures did you see? Why is confidently clearing a machine just as important as finding a fault?

- The two undercharge machines: what was similar between them? What, if anything, was different in how the fault presented?

Technical Discussion

- Dehydration: why does evacuation depth matter? What happens if refrigerant is added over residual moisture? What does a micron gauge tell you that a compound gauge cannot?

- Recovery compliance: EPA 608 obligations, recovery cylinder handling, when is a cylinder too full, and proper documentation.

- Recharge verification: how do you know the charge is correct? What are superheat and subcooling telling you, and how do they differ by system type?

Field Application

- What would change in the field if the machine had a hydraulic-driven cab vs. an engine-driven compressor?

- What would you do first if a machine came in with no cooling and the operator says 'it just stopped working overnight'?

- When is it appropriate to condemn a component vs. attempt repair?

Close-Out

- Open Q&A — anything from either day.

- Collect and review tech worksheets. Confirm all recovery and recharge quantities are logged.

- Remind techs: machines must receive full-duration evacuation before returning to field service.

- Paperwork, certifications, and sign-offs.