R-134a PT CHART  —  5,000 ft ELEVATION  |  Page 1 of 2

Local atm: 12.23 psia  |  Max vacuum: 24.9 in/Hg  |  PSIG = gauge pressure (unchanged by elevation)  |  Parentheses = sub-atmospheric

| 🔴 Vacuum — below 0 PSIG | 🔵 Low Side: 25–45 PSIG typical (40–80°F sat.) | 🟡 High Side: 116–180 PSIG typical at 5,000 ft | TXV system → use SUBCOOLING ¦ Target 10–20°F | Fixed Orifice → use SUPERHEAT ¦ Target 8–15°F |
| --- | --- | --- | --- | --- |

| Zone | Temp (°F) | PSIG | Vacuum (in/Hg) | 5,000 ft Notes |
| --- | --- | --- | --- | --- |
| Vacuum | -40°F | (14.7) | 29.9 in/Hg |  |
| Vacuum | -20°F | (10.2) | 20.8 in/Hg |  |
| Vacuum | -15°F | (8.9) | 18.1 in/Hg |  |
| Vacuum | -10°F | (7.4) | 15.1 in/Hg |  |
| Vacuum | -5°F | (5.8) | 11.8 in/Hg |  |
| Vacuum | 0°F | (4.1) | 8.3 in/Hg |  |
| Vacuum | 5°F | (2.2) | 4.5 in/Hg |  |
| Vacuum | 10°F | (0.3) | 0.6 in/Hg |  |
| Low Side | 14°F | 0.0 | — | 0 PSIG at sea level ref. (14.7 psia local atm) |
| Low Side | 15°F | 0.4 | — |  |
| Low Side | 20°F | 2.7 | — |  |
| Low Side | 21°F | 3.1 | — | ★ Local atmospheric at 5,000 ft — gauge ~3 PSIG but system is near local ambient pressure |
| Low Side | 25°F | 5.1 | — |  |
| Low Side | 30°F | 7.7 | — |  |
| Low Side | 35°F | 10.5 | — |  |
| Low Side | 40°F | 13.5 | — | Typical low side sat. temp — normal evaporator operating range begins here |
| Low Side | 45°F | 16.7 | — |  |
| Low Side | 50°F | 20.1 | — | Low side upper limit — higher readings suggest overcharge or airflow restriction |
| Low Side | 55°F | 23.8 | — |  |
| Low Side | 60°F | 27.7 | — |  |
| Low Side | 65°F | 31.9 | — |  |
| Low Side | 70°F | 36.4 | — |  |
| Low Side | 75°F | 41.1 | — |  |
| Low Side | 80°F | 46.2 | — |  |
| Low Side | 85°F | 51.6 | — |  |
| Low Side | 90°F | 57.3 | — |  |

R-134a PT CHART  —  5,000 ft ELEVATION  |  Page 2 of 2

Local atm: 12.23 psia  |  Max vacuum: 24.9 in/Hg  |  PSIG = gauge pressure (unchanged by elevation)  |  Parentheses = sub-atmospheric

| 🔴 Vacuum — below 0 PSIG | 🔵 Low Side: 25–45 PSIG typical (40–80°F sat.) | 🟡 High Side: 116–180 PSIG typical at 5,000 ft | TXV system → use SUBCOOLING ¦ Target 10–20°F | Fixed Orifice → use SUPERHEAT ¦ Target 8–15°F |
| --- | --- | --- | --- | --- |

| Zone | Temp (°F) | PSIG | Vacuum (in/Hg) | 5,000 ft Notes |
| --- | --- | --- | --- | --- |
| Low Side | 95°F | 63.4 | — |  |
| Low Side | 100°F | 69.8 | — |  |
| Low Side | 105°F | 76.6 | — |  |
| Low Side | 110°F | 83.7 | — |  |
| Low Side | 115°F | 91.3 | — |  |
| Low Side | 120°F | 99.2 | — |  |
| Low Side | 125°F | 107.5 | — |  |
| High Side | 130°F | 116.2 | — | High side lower limit at 5,000 ft — expect 10–20 PSIG lower than sea-level norms |
| High Side | 135°F | 125.4 | — |  |
| High Side | 140°F | 135.0 | — |  |
| High Side | 145°F | 145.1 | — |  |
| High Side | 150°F | 155.6 | — |  |
| High Side | 155°F | 166.6 | — |  |
| High Side | 160°F | 178.1 | — | Typical high side sat. temp at 5,000 ft in summer ambient conditions |
| High Side | 165°F | 190.2 | — |  |
| High Side | 170°F | 202.7 | — |  |
| High Side | 175°F | 215.8 | — |  |
| High Side | 180°F | 229.4 | — |  |
| High Side | 185°F | 243.6 | — |  |
| High Side | 190°F | 258.3 | — | High side caution — verify condenser airflow and ambient temperature |
| High Side | 195°F | 273.7 | — |  |
| High Side | 200°F | 289.6 | — |  |
| High Side | 205°F | 306.1 | — |  |
| High Side | 210°F | 323.3 | — |  |
| High Side | 215°F | 341.1 | — |  |
| High Side | 220°F | 359.6 | — |  |

| Superheat Calculation 1. Read suction (low side) PSIG 2. Find PSIG in chart → get sat. temp (°F) 3. Clamp probe on suction line → get actual temp 4. Superheat = actual temp − sat. temp Target: 8–15°F (fixed orifice systems) At 5,000 ft: suction pressures run slightly lower than sea-level norms for the same evaporator temp — this is normal. | Subcooling Calculation 1. Read discharge (high side) PSIG 2. Find PSIG in chart → get sat. temp (°F) 3. Clamp probe on liquid line → get actual temp 4. Subcooling = sat. temp − actual temp Target: 10–20°F (TXV systems) At 5,000 ft: head pressure runs 10–20 PSIG lower than sea level for the same ambient — judge by subcooling, not pressure. | 5,000 ft Key Differences Local atm: 12.23 psia (sea level: 14.70 psia) Max vacuum: ~24.9 in/Hg (sea level: 29.92 in/Hg) — a pump reading 25 in/Hg here is working correctly. Head pressure runs lower — thinner air means the condenser rejects heat more easily. This is normal. Do NOT overcharge to chase sea-level pressures. Use superheat/subcooling to verify charge — not raw pressure numbers. |
| --- | --- | --- |

R-134a  |  5,000 ft elevation (12.23 psia)  |  PSIG values are gauge pressure and do not change with elevation — absolute pressure and max vacuum depth do