UNDERSTANDING SUPERHEAT & SUBCOOLING A Technician's Guide to Reading Your Gauges and Diagnosing the Expansion Valve

Section 1 — The Big Picture

Before we get into the numbers, here is what’s actually happening inside your A/C system. Refrigerant travels in a continuous loop through four main parts:

| 1 COMPRESSOR Pumps refrigerant around the loop. Turns low-pressure vapor into high-pressure vapor. | 2 CONDENSER Sits outside. Hot vapor gives up heat to outside air and turns back into liquid. | 3 TXV / METERING Controls how much liquid refrigerant enters the evaporator. Drops the pressure suddenly. | 4 EVAPORATOR Sits inside the cab. Low-pressure liquid boils and absorbs heat — this is what cools the air. |
| --- | --- | --- | --- |

Superheat is measured on the LOW side — coming out of the evaporator. Subcooling is measured on the HIGH side — coming out of the condenser. Together they tell you if the right amount of refrigerant is circulating and if the expansion valve is doing its job.

Section 2 — Superheat

Refrigerant boils inside the evaporator coil. When it has fully boiled and turned to vapor, any extra heat added after that point is called superheat. It is measured in degrees Fahrenheit above the refrigerant's boiling point at the current low side pressure.

Plain english version: Imagine a pot of water on a stove. Water boils at 212°F. If you keep the heat on after it has all turned to steam, that steam gets hotter than 212°F. Those extra degrees above 212°F are superheat. Your refrigerant works exactly the same way — except it boils at a much lower temperature because of the low pressure inside the evaporator.

Why superheat matters

- If superheat is too LOW (near zero): liquid refrigerant is still in the suction line. Liquid can reach the compressor. Compressors only pump vapor — liquid will destroy a compressor fast. This is called slugging.

- If superheat is too HIGH (above 15°F): the evaporator is starved. Not enough refrigerant is boiling inside the coil, so the cab is not cooling efficiently.

- If superheat is in the normal range (8–15°F): the evaporator is fully fed and the compressor is protected.

How to measure superheat — step by step

- Connect your manifold gauges. Read the LOW SIDE (suction) pressure.

- Find that pressure on your R-134a PT chart. Look at the temperature column — that is your saturation temperature. Write it down.

- Clamp your temperature probe on the suction line (the large, cold line) as close to the evaporator outlet as you can get.

- Subtract:   Actual line temp  −  Saturation temp  =  Superheat

Worked Example — Calculating Superheat Your low side gauge reads: 40 PSIG You look up 40 PSIG on the PT chart → saturation temp: 50°F You clamp your probe on the suction line → actual line temp: 65°F Superheat = 65°F − 50°F = 15°F Result: 15°F superheat — right at the top of the normal range. System is okay.

Worked Example — High Superheat (problem found) Your low side gauge reads: 25 PSIG You look up 25 PSIG on the PT chart → saturation temp: 25°F You clamp your probe on the suction line → actual line temp: 55°F Superheat = 55°F − 25°F = 30°F Result: 30°F superheat — way too high. Evaporator is starved. Suspect undercharge or a restriction at the TXV.

Section 3 — Subcooling

After the condenser turns refrigerant vapor back into liquid, subcooling measures how many degrees below its condensing point that liquid has been cooled before it reaches the expansion valve.

Why does this matter? The expansion valve can only work correctly if it receives 100% liquid refrigerant. If there are any vapor bubbles mixed into the liquid line — called flash gas — the valve cannot control the flow properly and the system loses cooling capacity. Subcooling is your proof that the refrigerant arrived at the valve as pure liquid.

Why subcooling matters

- If subcooling is too LOW (near zero): flash gas is present. The system is short on charge, or the condenser is not cooling the liquid enough. The TXV will struggle and cooling will be poor.

- If subcooling is too HIGH (above 20°F): too much refrigerant is backed up in the condenser. This usually means overcharge or a restriction somewhere downstream of the condenser.

- If subcooling is in the normal range (10–20°F): pure liquid is reaching the valve and the high side is healthy.

How to measure subcooling — step by step

- Read the HIGH SIDE (discharge) pressure on your manifold gauges.

- Find that pressure on your PT chart. Look at the temperature column — that is your saturation temperature. Write it down.

- Clamp your temperature probe on the liquid line (the small, warm line leaving the condenser).

- Subtract:   Saturation temp  −  Actual line temp  =  Subcooling

Worked Example — Calculating Subcooling Your high side gauge reads: 160 PSIG You look up 160 PSIG on the PT chart → saturation temp: 113°F You clamp your probe on the liquid line → actual line temp: 98°F Subcooling = 113°F − 98°F = 15°F Result: 15°F subcooling — right in the middle of the normal range. Pure liquid is reaching the TXV.

Worked Example — Low Subcooling (problem found) Your high side gauge reads: 130 PSIG You look up 130 PSIG on the PT chart → saturation temp: 100°F You clamp your probe on the liquid line → actual line temp: 99°F Subcooling = 100°F − 99°F = 1°F Result: 1°F subcooling — flash gas is present in the liquid line. System is undercharged. Do not start diagnosing the TXV until charge is corrected.

Section 4 — Quick Reference

| Reading | Too Low | Normal Range | Too High |
| --- | --- | --- | --- |
| Superheat | Liquid in suction line — compressor damage risk | 8–15°F | Evaporator starved — undercharge or restriction |
| Subcooling | Flash gas in liquid line — undercharge | 10–20°F | Overcharge or downstream restriction |

Important rule before diagnosing the TXV: Always verify the charge is correct first. Low subcooling means low charge — fix the charge before you blame the valve. A starved TXV looks exactly like a broken TXV if the system is undercharged.

Section 5 — Diagnosing the Expansion Valve (TXV)

The thermostatic expansion valve (TXV) is a self-regulating valve. Its one job is to meter exactly the right amount of refrigerant into the evaporator to maintain a steady superheat — usually around 8–12°F. When it is working correctly, superheat stays stable even as conditions change.

This self-regulation is also what makes TXV diagnosis tricky. Because the valve compensates automatically, superheat alone will not tell you if the system is undercharged — the valve just closes down a little more and superheat stays normal. That is why subcooling is your charge indicator on TXV systems, not superheat.

The golden rule for TXV systems: Use SUBCOOLING to check the charge. Use SUPERHEAT to check the valve. If subcooling is normal and superheat is abnormal — the valve is your problem.

TXV Fault Diagnosis — What the Numbers Tell You

| Condition | Superheat | Subcooling |
| --- | --- | --- |
| TXV stuck open (flooding) | Very low — near 0°F or negative | Low — system is low on charge or valve won't close |
| TXV stuck closed (starving) | Very high — 20°F+ | Low — evaporator gets almost no refrigerant |
| TXV hunting (cycling rapidly) | Swings up and down constantly | Swings up and down — pressures unstable |
| Correct charge, TXV working | Steady 8–12°F | Steady 10–20°F |
| Undercharge, TXV compensating | Still looks normal (8–12°F) — TXV masks it | Low — this is how you catch it |

Worked Example — Catching an Undercharge Through Subcooling

Scenario: Tech checks a machine with poor cooling. Superheat looks fine. Low side: 35 PSIG → sat. temp 35°F Suction line temp: 46°F Superheat = 46 − 35 = 11°F ← looks normal. TXV is compensating. High side: 125 PSIG → sat. temp 95°F Liquid line temp: 94°F Subcooling = 95 − 94 = 1°F ← nearly zero. Flash gas present. Conclusion: System is undercharged. The TXV masked it on the superheat reading. Subcooling caught it. Add refrigerant to correct charge, then re-verify both readings.

Worked Example — TXV Stuck Closed

Scenario: Machine barely cools. Both pressures are low. Low side: 18 PSIG → sat. temp 18°F Suction line temp: 58°F Superheat = 58 − 18 = 40°F ← very high. Evaporator is starved. High side: 155 PSIG → sat. temp 112°F Liquid line temp: 96°F Subcooling = 112 − 96 = 16°F ← normal. Charge is fine. Conclusion: Charge is correct (subcooling is good) but superheat is extremely high. The TXV is not feeding the evaporator — stuck closed or restricted. Replace or inspect the TXV.

Worked Example — TXV Stuck Open (Flooding)

Scenario: Machine runs but compressor is making a knocking sound. Low side: 55 PSIG → sat. temp 60°F Suction line temp: 58°F Superheat = 58 − 60 = −2°F ← negative. Liquid in the suction line. High side: 140 PSIG → sat. temp 106°F Liquid line temp: 89°F Subcooling = 106 − 89 = 17°F ← normal. Charge is fine. Conclusion: Charge is correct but superheat is negative — liquid refrigerant is flooding back to the compressor. TXV is stuck open or the sensing bulb has lost its charge. The knocking sound is liquid slugging. Shut down immediately and inspect the TXV.

Section 6 — Field Checklist

Follow this order every time. Do not skip ahead to valve diagnosis until charge is confirmed.

| ☐ | Connect manifold gauges. Let system stabilize for 5 minutes before reading. |
| --- | --- |
| ☐ | Record low side pressure → look up saturation temp on PT chart. |
| ☐ | Clamp probe on suction line → calculate SUPERHEAT. |
| ☐ | Record high side pressure → look up saturation temp on PT chart. |
| ☐ | Clamp probe on liquid line → calculate SUBCOOLING. |
| ☐ | Check subcooling first. If it is below 10°F — correct the charge before anything else. |
| ☐ | If subcooling is normal but superheat is abnormal — the TXV is your suspect. |
| ☐ | Use the TXV fault table above to identify the specific valve condition. |
| ☐ | Document all readings on your worksheet: pressures, line temps, superheat, subcooling. |

Remember Subcooling tells you about the CHARGE. Superheat tells you about the VALVE. Fix the charge first. Then diagnose the valve. Never the other way around.